Craven Local Plan: Residential Site Selection Process

Settlement Name: Bentham

Site Number and Address: HB025; Rear of 38-54 Robin Lane and east of Butts Lane

Stage 1: Settlement Level Analysis (Sustainability Appraisal)

Result: Site performs satisfactorily in the Sustainability Analysis. The site is deemed sustainable in order to enter the Pool of Sites (with mitigation measures and recommendations). The site is a candidate for the Preferred Sites list for High Bentham, subject to Stage 2 analysis below, and also a possible comparison of the site’s merits with other sustainable sites in High Bentham if a surplus of sites exists.

Stage 2: District Level Analysis

- Viability of Affordable Housing Provision

| - The site is likely to deliver 11 or more dwellings (irrespective of being in a designated rural area or not). Affordable housing on site is obtained. | Yes |
| --- | --- |
| - The site is in a designated rural area, and is likely to deliver 6 or more dwellings. A financial contribution towards affordable housing is obtained. |  |
| - The site is unlikely to deliver affordable housing (either less than 11 dwellings is likely and the site is not in a designated rural area, or less than six dwellings is likely in a designated rural area). |  |

- Site Proximity to Designated Landscape Features

| - Site is likely to have potential to harm the special qualities of the designated landscapes of the YDNP or the AONB, and any harm cannot be mitigated against. | No |
| --- | --- |
| - The location of the site in the context of the settlement (i.e. north-west, central, south-east, etc.; this gives an indication of relative proximity to YDNP or AONB, whichever is the nearest). | North-east |

- Site Proximity to Designated Natural Environment Features

| - Site is likely to have potential to harm the special qualities of the designated landscapes of the SPA, SAC or Ramsar, and any harm cannot be mitigated against. | No |
| --- | --- |
| - The location of the site in the context of the settlement (i.e. north-west, central, south-east, etc.; this gives an indication of relative proximity to a SPA, SAC or Ramsar, whichever is the nearest). | North-east |

- Site Position in relation to Health & Safety Executive (HSE) Zone

| - The site lies in the outer Health & Safety Executive zone, or not in an HSE zone. | Yes |
| --- | --- |
| - The site lies in the Health & Safety Executive middle zone. |  |
| - The site lies in the Health & Safety Executive inner zone. |  |

Result: The site passes all four District Level Analyses. After a comparison with other Pool of Site options in High Bentham, the site is considered suitable as one of the Preferred Sites in this settlement. [If a Preferred Site after comparison, explain in the narrative below, and if not a Preferred Site after comparison, explain comparative reasons here in the context of the settlement’s chosen Preferred Sites]

Narrative of Site Analysis

HB025 returns a mixture of positive, negative and neutral results in the analysis. Advantages of the site include nil impact on heritage assets. Flood risk is low on the southern part of the site, with limited surface water risk along the eastern site boundary. The northern part is subject to more extensive surface water flood risk. The risk of groundwater emergence is medium throughout. Safe road access cannot be provided from Robin Lane, but can be provided from Butts Lane into the southern part of the site. The site is located on the edge of the settlement, in a fairly prominent position, where development may have a greater impact on the wider landscape and where town centre services and facilities are more distant. Land within the site is also of greater agricultural value (grade 3). Omission of the northern part of the site from any allocation would provide significant mitigation against these drawbacks and further mitigation should be possible on the southern part of the site through good design, layout, landscaping and SuDS.

Approx. Area: 1.0 hectares

Approx. Number of Dwellings: 32

Development Principles:

- A surface water flood risk hazard has been identified along the eastern site boundary. Development proposals must therefore be supported by a Flood Risk Assessment and drainage strategy which has informed the design, layout and landscaping of the site. Proposals will incorporate Sustainable Drainage Systems (SuDS), where possible;

- The site’s prominent location will be addressed in the design, layout and landscaping of the development to ensure that the character and appearance of the local area is not adversely affected and will include measures to minimise impacts on air quality, noise and light pollution;

- The design of proposals shall conserve the landscape and scenic beauty of the Forest of Bowland AONB;

- Access to the site will be gained from Butts Lane;

- Development proposals for this site must accord with local plan policies H2, INF3 and INF6 (which set out requirements for contributions towards affordable housing, education provision and sport, open space and recreation facilities) and all other relevant local plan policies.