Craven Local Plan: Residential Site Selection Process

Settlement Name: Bradley

Site Number and Address: BR016; Land to the west of Gilders and Langholme, Skipton Road

Stage 1: Settlement Level Analysis (Sustainability Appraisal)

| BR016: Land to the west of Gilders and Langholme, Skipton Road; 1.28ha. | 0 | 0 | + | + | + | + | 0 | + | ++ | 0 | 0 | 0 | + | - | + | 0 | + | + | + | - | Level 3: Sufficient frontage is available on site to allow for a safe access. The site is entirely within Flood Zone 1. To ensure the sustainability of the site in terms of connectivity to the village, it is recommended that the southern section of the site is developed only. The site provides a strong contribution to the character and appearance of the area, to the south east of the site it provides some or no/negligible contribution to the character and appearance of the area. The site is located on the edge of the settlement and is greenfield land. | 5a, 5b, 7, & 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Site performs satisfactorily in the Sustainability Analysis. The site is deemed sustainable in order to enter the Pool of Sites (with mitigation measures and recommendations). The site is a candidate for the Preferred Sites list for Bradley, subject to Stage 2 analysis below, and also a possible comparison of the site’s merits with other sustainable sites in Bradley if a surplus of sites exists.

Stage 2: District Level Analysis

- Viability of Affordable Housing Provision

| - The site is likely to deliver 11 or more dwellings (irrespective of being in a designated rural area or not). Affordable housing on site is obtained. | Yes |
| --- | --- |
| - The site is in a designated rural area, and is likely to deliver 6 or more dwellings. A financial contribution towards affordable housing is obtained. |  |
| - The site is unlikely to deliver affordable housing (either less than 11 dwellings is likely and the site is not in a designated rural area, or less than six dwellings is likely in a designated rural area). |  |

- Site Proximity to Designated Landscape Features

| - Site is likely to have potential to harm the special qualities of the designated landscapes of the YDNP or the AONB, and any harm cannot be mitigated against. | No |
| --- | --- |
| - The location of the site in the context of the settlement (i.e. north-west, central, south-east, etc.; this gives an indication of relative proximity to YDNP or AONB, whichever is the nearest). | North |

- Site Proximity to Designated Natural Environment Features

| - Site is likely to have potential to harm the special qualities of the designated landscapes of the SPA, SAC or Ramsar, and any harm cannot be mitigated against. | No |
| --- | --- |
| - The location of the site in the context of the settlement (i.e. north-west, central, south-east, etc.; this gives an indication of relative proximity to a SPA, SAC or Ramsar, whichever is the nearest). | North |

- Site Position in relation to Health & Safety Executive (HSE) Zone

| - The site lies in the outer Health & Safety Executive zone, or not in an HSE zone. | Yes |
| --- | --- |
| - The site lies in the Health & Safety Executive middle zone. |  |
| - The site lies in the Health & Safety Executive inner zone. |  |

Result: The site passes all four District Level Analyses. After a comparison with other Pool of Site options in Bradley, the site is considered suitable as one of the Preferred Sites in this settlement.

Narrative of Site Analysis

Site BR016 has numerous positive aspects which have determined the site’s selection as a Preferred Site. The site is adjacent to the village centre and has good access and sight lines in both directions. The site also performs satisfactorily regarding the District level analysis.  This site also conforms with the choice of the draft Bradley Neighbourhood Plan.

Considering the rising land to the north of the site and the current level of housing need in Bradley, the northern section of the submitted site is not needed.

Approximate Area: 0.8 hectares (southern part of available SHLAA site).

Approximate Number of Dwellings: 25 dwellings

Development Principles:

- Siting and design of development on the site to conserve the setting of the Bradley Conservation Area;

- The site is in a prominent location. Development proposals should be carefully and sensitively designed to minimise visual impact on the character and appearance of the area, and include measures to minimise impacts on air quality, noise and light pollution;

- An assessment of the site’s archaeological interest will be required with appropriate mitigation incorporated into development proposals where necessary;

- The existing dry stone boundary walls will be retained. New dry stone boundary walls will be created along the northern and western boundaries of the site to help establish a definitive new urban edge to the village;

- Access to the site is to be gained from Skipton Road;

- Development proposals for this site must accord with local plan policies H2, INF3 and INF6 (which set out requirements for contributions towards affordable housing, education provision and sport, open space and recreation facilities) and all other relevant local plan policies.