Craven Local Plan: Residential Site Selection Process

Settlement Name: Langcliffe

Site Number and Address: LA004; Land west of Barrel Sykes.

Stage 1: Settlement Level Analysis (Sustainability Appraisal)

Result: Site performs satisfactorily in the Sustainability Analysis. The site is deemed sustainable in order to enter the Pool of Sites (with mitigation measures and recommendations). The site is a candidate for the Preferred Sites list for Settle, subject to Stage 2 analysis below, and also a possible comparison of the site’s merits with other sustainable sites in Settle if a surplus of sites exists.

Stage 2: District Level Analysis

- Viability of Affordable Housing Provision

| - The site is likely to deliver 11 or more dwellings (irrespective of being in a designated rural area or not). Affordable housing on site is obtained. | Yes |
| --- | --- |
| - The site is in a designated rural area, and is likely to deliver 6 or more dwellings. A financial contribution towards affordable housing is obtained. |  |
| - The site is unlikely to deliver affordable housing (either less than 11 dwellings is likely and the site is not in a designated rural area, or less than six dwellings is likely in a designated rural area). |  |

- Site Proximity to Designated Landscape Features

| - Site is likely to have potential to harm the special qualities of the designated landscapes of the YDNP or the AONB, and any harm cannot be mitigated against. | No |
| --- | --- |
| - The location of the site in the context of the settlement (i.e. north-west, central, south-east, etc.; this gives an indication of relative proximity to YDNP or AONB, whichever is the nearest). | North |

- Site Proximity to Designated Natural Environment Features

| - Site is likely to have potential to harm the special qualities of the designated landscapes of the SPA, SAC or Ramsar, and any harm cannot be mitigated against. | No |
| --- | --- |
| - The location of the site in the context of the settlement (i.e. north-west, central, south-east, etc.; this gives an indication of relative proximity to a SPA, SAC or Ramsar, whichever is the nearest). | North |

- Site Position in relation to Health & Safety Executive (HSE) Zone

| - The site lies in the outer Health & Safety Executive zone, or not in an HSE zone. | Yes |
| --- | --- |
| - The site lies in the Health & Safety Executive middle zone. |  |
| - The site lies in the Health & Safety Executive inner zone. |  |

Result: The site passes all four District Level Analyses. After a comparison with other Pool of Site options in Settle, the site is considered suitable as one of the Preferred Sites in this settlement.

Narrative of Site Analysis

Site LA004 has numerous positive aspects which have determined the site’s selection as a Preferred Site.  The site is situated directly to the north of the town’s built up area, which allows easy access to the town’s services, and hence improving connectivity and reducing the need for travel. The site has an existing, safe access with good sightlines available. The site also performs satisfactorily regarding the District level analysis.

The site has been identified in the Settle Carlisle Conservation Area appraisal as having an impact on the conservation area to the east.  As urban form is not wholly alien to this railway conservation area, the development of the site would not have a huge adverse impact on this heritage asset.  The site is also in close proximity to the grade II listed Watershed Mill to the west, which should be afforded protection by keeping development to the south of mill chimney so as to retain an open countryside aspect across from these important heritage assets.  As such the southern section of the site area submitted to the SHLAA database can be utilised.

Approximate Site Area: 0.6 hectares (southern section of available SHLAA site).

Approximate Number of Dwellings: 19 dwellings

Development Principles:

- There is a requirement for a Biodiversity Appraisal to assess the existing ecological conditions on the site.  This is to be accompanied by a standardised Biodiversity Mitigation Plan (BMP) which must cover matters of both habitat and protected species, with the aim that negative effects on biodiversity are avoided or suitably offset, and enhancement effects are implemented;

- Siting and design of development on the site will conserve the significance of the heritage assets (Settle Carlisle Railway Conservation Area and Watershed Mill, Chimney and Shed Mill Cottages) adjacent to the site and their settings. Proposals will retain the views across this site from the Settle Carlisle Conservation Area towards Watershed Mill;

- As the site is in a prominent location, development proposals will be carefully and sensitively designed to minimise visual impact on the character and appearance of the area, and include measures to minimise impacts on air quality, noise and light pollution. Dwelling heights will be restricted to two storeys to protect views from the Settle Carlisle Railway Conservation Area towards Watershed Mill.  Dwellings on the western boundary will be front-facing, but set back from Langcliffe Road to ensure an attractive entrance to this northern part of the town;

- The existing dry stone boundary walls will be retained. A new dry stone boundary wall will be created east to west across the northern boundary of the site to enclose the field to the north of the site opposite Watershed Mill and to help establish a definitive new urban edge to the town;

- A surface water flood risk hazard has been identified within the southern part of the site.   Development proposals must therefore be supported by a Flood Risk Assessment and drainage strategy which has informed the design, layout and landscaping of the site. Proposals will incorporate Sustainable Drainage Systems (SuDS), where possible;

- Access to the site is to be gained from Barrel Sykes;

- Development proposals for this site must accord with local plan policies H2, INF3 and INF6 (which set out requirements for contributions towards affordable housing, education provision and sport, open space and recreation facilities) and all other relevant local plan policies.