Craven Local Plan: Residential Site Selection Process

Settlement Name: Settle

Site Number and Address:  SG079; Land to the north of Town Head Way, Settle

Stage 1: Settlement Level Analysis (Sustainability Appraisal)

Result: Site performs satisfactorily in the Sustainability Analysis. The site is deemed sustainable in order to enter the Pool of Sites (with mitigation measures and recommendations). The site is a candidate for the Preferred Sites list for Settle, subject to Stage 2 analysis below, and also a possible comparison of the site’s merits with other sustainable sites in Settle if a surplus of sites exists.

Stage 2: District Level Analysis

- Viability of Affordable Housing Provision

| - The site is likely to deliver 11 or more dwellings (irrespective of being in a designated rural area or not). Affordable housing on site is obtained. | Yes |
| --- | --- |
| - The site is in a designated rural area, and is likely to deliver 6 or more dwellings. A financial contribution towards affordable housing is obtained. |  |
| - The site is unlikely to deliver affordable housing (either less than 11 dwellings is likely and the site is not in a designated rural area, or less than six dwellings is likely in a designated rural area). |  |

- Site Proximity to Designated Landscape Features

| - Site is likely to have potential to harm the special qualities of the designated landscapes of the YDNP or the AONB, and any harm cannot be mitigated against. | No |
| --- | --- |
| - The location of the site in the context of the settlement (i.e. north-west, central, south-east, etc.; this gives an indication of relative proximity to YDNP or AONB, whichever is the nearest). | North-east |

- Site Proximity to Designated Natural Environment Features

| - Site is likely to have potential to harm the special qualities of the designated landscapes of the SPA, SAC or Ramsar, and any harm cannot be mitigated against. | No |
| --- | --- |
| - The location of the site in the context of the settlement (i.e. north-west, central, south-east, etc.; this gives an indication of relative proximity to a SPA, SAC or Ramsar, whichever is the nearest). | North-east |

- Site Position in relation to Health & Safety Executive (HSE) Zone

| - The site lies in the outer Health & Safety Executive zone, or not in an HSE zone. | Yes |
| --- | --- |
| - The site lies in the Health & Safety Executive middle zone. |  |
| - The site lies in the Health & Safety Executive inner zone. |  |

Result: The site passes all four District Level Analyses. After a comparison with other Pool of Site options in Settle, the site is considered suitable as one of the Preferred Sites in this settlement.

Narrative of Site Analysis

Site SG079 has numerous positive aspects which have determined the site’s selection as a Preferred Site.  The site is situated directly to the north-east of the town’s built up area, which allows easy access to the town’s services, and hence improving connectivity and reducing the need for travel. The site has an existing, safe access with good sightlines available. The site also performs satisfactorily regarding the District level analysis.

The site is within close proximity to the National Park boundary to the east and has been identified in the Settle Carlisle Conservation Area appraisal as having an impact on the conservation area to the west.  As urban form is not wholly alien to this railway conservation area, the development of the site would not have a huge adverse impact on this heritage asset.  Site development could have the added benefit of restoring and opening up the tunnel through the embankment immediately to the west of the west boundary of the site.  This tunnel is an important feature of the railway but is almost hidden from view at present and is not used.  Opening up and restoring the tunnel would enhance the heritage asset of the tunnel and provide much improved pedestrian permeability between each side of the railway.   Considering the proximity of the National Park to the east, development on site should be set back from the National Park boundary and include an area of green infrastructure to protect this part of the site from future development.  Considering the proximity of the grade II listed Barrel Sykes Farmhouse to the north, development on site should be set back from the northern boundary and include an area of green infrastructure, enclosed by dry stone walls to mimic existing field patterns to the north at Barrel Sykes Farm.  These areas of green infrastructure will also provide footpath links to the railway tunnel footpath to the north and the National Park highway/footpath and PROW to the east and recreation mitigation for the Ingleborough Complex Special Area of Conservation (SAC) and Craven Limestone Complex Special Area of Conservation (SAC).

Approximate Site Area: 1.7 hectares (all of the available SHLAA site).

Approximate Number of Dwellings: 26 dwellings

Development Principles for SG079:

- This is a greenfield site in a prominent location on the edge of Settle. Development proposals for this site will incorporate landscape and recreation mitigation(s) including a green infrastructure area of approximately 0.9 ha along the eastern and northern boundaries of the site to provide landscape mitigation for the Yorkshire Dales National Park and the adjacent grade II listed building, Barrel Sykes Farm.  The areas of green infrastructure will also deliver recreational walking opportunities aimed at relieving pressure on the Ingleborough Complex Special Area of Conservation (SAC) and Craven Limestone Complex Special Area of Conservation (SAC) by providing footpath links to the railway tunnel footpath to the north and the National Park highway/footpath and PROW to the east. The layout of the site will be designed to leave gaps through the site to retain views from the National Park to the Grade II listed Barrel Sykes Farm and the undesignated heritage asset of Watershed Mill chimney beyond. The layout of the site will also ensure that views from Town Head Way north towards Barrel Sykes Farm and the Watershed Mill chimney are retained;

- Siting and design of development on the site to conserve the significance of the adjacent Settle-Carlisle Railway Conservation Area to the west and the grade II listed Barrel Sykes Farm to the north;

- The existing dry stone boundary walls will be retained. A new dry stone boundary wall will be created east to west across the site to enclose the northern area of green infrastructure in order to maintain the setting of the adjacent heritage asset of the grade II listed Barrel Sykes Farm and to provide a clear definable edge to the development;

- Development proposals on site will include an improved and enhanced pedestrian link from the site via the adjacent tunnel through the railway embankment;

- A Landscape Visual Impact Assessment (LVIA) is required to assess the likely effects of change on the landscape as a result of the development, specifically on views into and out of the Yorkshire Dales National Park. The LVIA will help locate and design the development so that negative landscape effects are avoided, appropriately reduced or offset;

- There is a requirement for a Biodiversity Appraisal to assess the existing ecological conditions on the site.  This is to be accompanied by a standardised Biodiversity Mitigation Plan (BMP) which must cover matters of both habitat and protected species, with the aim that negative effects on biodiversity are avoided or suitably offset, and enhancement effects are implemented;

- As the site is in a prominent location, development proposals will be carefully and sensitively designed to minimise visual impact on the character and appearance of the area, and include measures to minimise impacts on air quality, noise and light pollution;

- A surface water flood risk hazard has been identified within the northern part of the site.   Development proposals must therefore be supported by a Flood Risk Assessment and drainage strategy which has informed the design, layout and landscaping of the site. Proposals will incorporate Sustainable Drainage Systems (SuDS), where possible;

- Access to the site is to be gained from Town Head Way;

- Development proposals for this site must accord with local plan policies H2, INF3 and INF6 (which set out requirements for contributions towards affordable housing, education provision and sport, open space and recreation facilities) and all other relevant local plan policies.